第 1 课	传输介质

- Which is one of the key features of Global Positioning System (GPS)?C

  - Accuracy about 2 kilometers	B. 1200 total satellites orbit the earth.

C. Provide accurate time information.	D. Open to American users only.

全球定位系统（GPS）的关键特征之一是提供准确的位置信息，而不是精度约2公里、地球轨道上共有1200颗卫星或仅对美国用户开放等选项。因此答案是C. 提供准确的时间信息。

- Which of the following energies can traverse long distances and penetrate objects such as the walls of a building?D

  - electrical	B. heat	C. light	D. radio

可以穿过长距离并穿透建筑物的墙壁的能量是无线电波。因此答案是D. 无线电波。

- Which of the following is NOT the form of energy used in computer network?B

  - electrical	B. heat	C. light	D. radio

不确定

- In a factory that uses electric arc welding equipment, （B） is the best wiring for short-distance communication.

  - unshielded twisted pair	B. shielded twisted pair

C. coaxial cable	D. optical fibers

在使用电弧焊接设备的工厂中，最适合用于短距离通信的布线是屏蔽的双绞线，因为它可以减少电磁干扰。因此答案是B. 屏蔽的双绞线。

- To reduce the interference caused by random electromagnetic radiation, communication systems DO NOT use （C）.

  - twisted pair wiring	B. parallel pair wiring

C. coaxial cable	D. none of above

为了减少随机电磁辐射引起的干扰，通信系统不使用同轴电缆，因为它无法消除电磁干扰。因此答案是C. 同轴电缆。

- Which of the following media is used in RJ-45 wire cable?A

  - twisted pair wire	B. parallel pair wire

C. coaxial cable	D. fiber

RJ-45网线电缆中使用的介质是双绞线，它由两根绝缘电线缠绕在一起以减少电磁干扰。因此答案是A. 双绞线。

- Communication to a satellite uses （C） energy.

  - light	B. heat	C. radio	D. electrical

通信卫星使用的是无线电信号进行通讯。在地球上，我们通过无线电发射器将信息传输到卫星上，卫星再通过无线电信号将信息传输到接收器上。

- Logically, twisted pair Ethernet employs a （A）topology. Physically, however, twisted pair Ethernet forms a （	）-shaped topology.

  - bus; star	B. star; bus	C. ring; star	D. star; ring

在逻辑上，双绞线以总线拓扑结构为主，即所有计算机都连接到同一根电缆上。但从物理上来说，由于我们常常使用交换机将计算机连接在一起，因此双绞线形成了星型拓扑结构。

- Optical fiber is more desirable than copper wiring, EXCEPT:C

  - Resistance to electrical noise.	B. Higher bandwidth.

C. Lower overall cost.	D. Less signal energy loss.

光纤比铜线更优秀的方面有：抗电噪声、更高的带宽、信号能量损失更小等。不过，光纤的制造成本和设备成本相对较高，这是铜线所占的优势之一。

- （B） may NOT be sufficient to run above the ceiling in an office building on top of a florescent light fixture.

  - twisted pair wiring	B. coaxial cable

C. shielded twisted pair	D. none of above

同轴电缆通常用于低电压、高频率的信号传输，但对于安装在天花板上的荧光灯装置上方来说，同轴电缆可能不够灵活，因此不适合使用。

- 10BASE-T uses （B） cable.

  - thick coaxial	B. unshielded twisted-pair

C. thin coaxial	D. fiber-optic

10BASE-T是一种以双绞线为基础的以太网标准，其中的“T”代表“双绞线”（Twisted Pair）。10BASE-T使用未屏蔽双绞线作为传输媒介，支持最高10 Mbps的数据传输速率。

- RJ-45 network cable with which we connect two computers uses （B） cable.

  - parallel-pair	B. twisted-pair	C. coaxial	D. fiber-optic

RJ-45网络电缆是一种常见的以太网电缆，用于连接计算机和局域网（LAN）中的其他设备。通常使用双绞线作为传输媒介。

- The （D） is not affected by electromagnetic interference or noise.

  - satellite	B. twisted-pair	C. coaxial	D. optical fiber

光纤缆（optical fiber）不受电磁干扰或噪声的影响。这是因为光纤缆是通过光信号传输数据，而不是通过电信号传输数据。光信号的传输速度快、抗干扰性强、信号衰减小，因此在一些高要求的通信场合中，如长距离通信和高速通信等方面有广泛应用。

- Which of the following can be used with especially strong electrical noise?B（不确定，可能是D）

  - parallel-pair	B. twisted-pair	C. coaxial	D. none of above

并行双绞线电缆和同轴电缆都是易受强电噪声影响的电缆。在高噪声环境下，这些电缆可能无法提供稳定和可靠的通信。因此，在这种情况下，这些电缆不适合使用。一般来说，双绞线电缆使用屏蔽层可以提高它的抗干扰性能，但并不能完全避免电磁干扰的影响。

- Which of the following technologies is used in 4G network?A

  - radio	B. twisted-pair	C. coaxial	D. optical

4G网络使用射频信号传输数据，这是一种无线技术。4G网络具有高速数据传输、较低延迟、更好的覆盖范围等优势，可以支持各种应用，如高清视频、在线游戏等。

- Which technology is used in the connection between a smart phone and a speaker?C

  - GPRS.	B. GPS.	C. Bluetooth.	D. Ethernet.

智能手机与扬声器之间的连接通常使用蓝牙技术。蓝牙是一种短距离无线通信技术，可以在设备之间传输数据，如音频、视频、图像和文本等。蓝牙技术适用于多种设备之间的连接，如手机、耳机、车载系统和智能家居设备等。

- 关于单模光纤，下面的描述中错误的是（C）。

  - 芯线由玻璃或塑料制成	B. 比多模光纤芯径小

C. 光波在芯线中以多种反射路径传播 D. 比多模光纤的传输距离远

单模光纤（single-mode fiber）的描述中错误的是“光波在芯线中以多种反射路径传播”。单模光纤中的光波只沿着一条路径传播，因此能够实现更高的传输速率和更远的传输距离。

第 2 课	局域异步通信

- RS-232 DB9 uses （C） transmission during the communication.

  - simplex	B. half duplex	C. full duplex	D. none

全双工

- The RS-232 standard sends each bit of the character, and follows each character with an idle period （C） bit(s) long.

  - no	B. one	C. at least one	D. at most one.

- Which is correct about RS-232-C?C

  - The connection must be less than 15 feet long.(好像不是必须？)

  - The voltage ranges from -50 volts to +50 volts.

  - A sender transmit an extra 1 bit (called a start bit) before transmitting a character

  - A sender must can send characters without leaving the line idle between them.

发送方在传输字符前要发送一个额外的1位（称为起始位）。RS-232-C标准规定发送方在每个字符之前必须发送一个起始位，以便接收方确定何时开始接收字符。

- Which of the following about RS-232-C is correct?A

  - RS-232-C specifies that a sender transmits a start bit before transmitting the bits of a character, and a stop bit is appended to each character.

  - When it finishes transmission, the sender leaves the wire with a positive voltage until another character is ready for transmission.

  - RS-232-C uses voltage ranging from -5 volts to +5 volts.

  - Negative voltage corresponds to logical 0, while positive voltage corresponds to logical 1.

A. RS-232-C规定发送方在传输字符的位之前发送一个起始位，并在每个字符后附加一个停止位。RS-232-C标准规定发送方在传输字符时必须发送一个起始位和一个停止位，以便接收方确定何时开始和结束接收字符。

- Which of the following about RS-232-C is WRONG?C

  - Each data item represents one character in keyboard.

  - Once transmission begin, a sender transmits a start bit, all bits of the characters, and there is one stop bit at the end of transmission.

  - The connection must be less than 50-feet long.

  - The hardware can be configured to control the exact number of bits per second.

RS-232-C 的连接长度可以远大于50英尺，但是连接长度越长，传输速率就越慢，同时容易出现噪声和数据传输错误。

- Which of the following about RS-232-C is WRONG?B

  - It is used for asynchronous, serial communication over short distances.

  - Negative voltage corresponds to logical 0, while positive voltage means logical 1.

  - A sender transmits all bits of the byte one after another with no delay between them.

  - RS-232-C uses voltage ranging from -15 volts to +15 volts.

- Which of the following standards has the HIGHEST bit rate?D

  - RS-232-C	B. 10BaseT	C. ASDL	D. OC-3

OC-3 标准具有最高的比特率，为155 Mbps。RS-232-C 标准的比特率最高为20 kbps，10BaseT 标准的比特率为10 Mbps，ASDL 标准的比特率为8 Mbps。

- Which of the following statement is WRONG?B

  - Computers CANNOT tell whether they are on a single segment or a bridged LAN.

计算机无法感知LAN的拓扑结构，无法区分单个网络段和桥接的LAN。

  - The RS-232-C connection must be less than 50-feet long.

  - ARP helps to translate a human-readable symbol to MAC address.

ARP (Address Resolution Protocol) is used to translate an IP address to a MAC address on a local network. ARP（地址解析协议）用于将IP地址解析为对应的MAC地址，以便网络设备能够相互通信。

  - A hub can only support one transmission at a time.

集线器是一种共享介质的网络设备，一次只能支持一个传输。如果多台设备同时尝试发送数据，将会发生冲突并导致数据包损坏。

B。RS-232-C 的连接长度可以远大于50英尺。